| # SAFETY OPERATING PROCEDURES Belt/Disk Sander |  |
| --- | --- |
| DO NOT use this machine unless you have been given permission |  |
| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained or restrained. |
| Appropriate protective footwear with substantial uppers must be worn. | Close fitting, protective clothing or a workshop apron is encouraged. |
| Rings and jewellery must not be worn. | DO NOT wear gloves when operating this sanding machine. |

### Only one person may operate this machine at any one time.

### PRE-OPERATIONAL SAFETY CHECKS

- Check the workroom and walkways to ensure there are no slip/trip hazards present.

- Ensure you are familiar with the operation of the ON/OFF switch.

- Check the sanding disc table is set not more than 2mm from disc.

- Check the linishing belts and discs are both in a serviceable condition.

- Ensure the operator be positioned out of direct line of abrasive belt at all times.

- Ensure dust extraction is on before operating either sanding machine.

- Faulty equipment must not be used. Immediately report suspect machinery.

OPERATIONAL SAFETY CHECKS

- Both components of this combination machine should NOT, where possible, be operated simultaneously.

- Allow machine to reach maximum revolutions before operating to avoid overloading.

- Always place material on the table on the downward side of the disc travel to hold it secure on the table surface or belt surface. Never attempt to sand very small items by hand.

- Hold material firmly against stops or table before applying pressure on abrasive disc or belt.

- Keep fingers clear of sanding disc or belt while sanding.

- NEVER attempt to sand very small items or try to sharpen metal items.

- Never leave the machine while it is running.

- Before making adjustments switch off and bring the machine to a complete standstill.

HOUSEKEEPING

1.   Switch off the machine and leave the floor area in a safe, clean and tidy state.

2.   Make sure good housekeeping practices are in place to minimise dust/waste build-up.

POTENTIAL HAZARDS

#### 	Exposure to moving, abrasive & rotating parts        Pinch & squash

#### 	Ejected waste 	     Excessive wood dust          Eye injuries          Burns to skin